#!/bin/sh

#

# Priority map:	Premium=1:10, Express=1:20, Standard=1:30, Bulk=1:40, Exempt=untouched

#

#DOWNLINK=$1

#UPLINK=$2

#DEV=$3

#MTU=$4

TC=/usr/sbin/tc

IPTABLES=/usr/sbin/iptables

IP=/usr/sbin/ip

if [ "$1" = "status" ]

then

$TC -s qdisc ls dev $2

$TC -s class ls dev $2

$TC -s qdisc ls dev imq0

$TC -s class ls dev imq0

exit

fi

# clean existing down- and uplink qdiscs, hide errors

$TC qdisc del dev $3 root    2> /dev/null > /dev/null

$TC qdisc del dev imq0 root  2> /dev/null > /dev/null

$IP link set imq0 down

#$TC qdisc del dev $3 ingress 2> /dev/null > /dev/null

if [ "$1" = "stop" ]

then

exit

fi

###### uplink

# install root HTB, point default traffic to 1:30:

$TC qdisc add dev $3 root handle 1: htb default 30

# main class - max device bandwidth

$TC class add dev $3 parent 1: classid 1:1 htb rate 100mbit burst 1000k quantum $((${4}+18))

# Exempt class 1:100

$TC class add dev $3 parent 1:1 classid 1:100 htb rate 100mbit ceil 100mbit burst 1000k prio 1 quantum $((${4}+18))

# root QoS class - max to pipe's max rate

$TC class add dev $3 parent 1:1 classid 1:2 htb rate $2kbit burst 6k prio 2 quantum $((${4}+18))

# Premium class 1:10

$TC class add dev $3 parent 1:2 classid 1:10 htb rate $((75*${2}/100))kbit ceil $2kbit burst 6k prio 3 quantum $((${4}+18))

# Express class 1:20

$TC class add dev $3 parent 1:2 classid 1:20 htb rate $((15*${2}/100))kbit ceil $2kbit burst 6k prio 4 quantum $((${4}+18))

# Standard class 1:30

$TC class add dev $3 parent 1:2 classid 1:30 htb rate $((10*${2}/100))kbit ceil $2kbit burst 6k prio 5 quantum $((${4}+18))

# Bulk class 1:40

$TC class add dev $3 parent 1:2 classid 1:40 htb rate 1kbit ceil $2kbit burst 6k prio 6 quantum $((${4}+18))

# add SFQ

$TC qdisc add dev $3 parent 1:100 handle 100: sfq perturb 10

$TC qdisc add dev $3 parent 1:10 handle 10: sfq perturb 10

$TC qdisc add dev $3 parent 1:20 handle 20: sfq perturb 10

$TC qdisc add dev $3 parent 1:30 handle 30: sfq perturb 10

$TC qdisc add dev $3 parent 1:40 handle 40: sfq perturb 10

## separate iptables mark into proper classes

## tc filter stops when it finds a match, so rules are reversed here

## in order to make sure we put bulk traffic in its place

$TC filter add dev $3 parent 1:0 protocol ip prio 2 handle 40 fw flowid 1:40

$TC filter add dev $3 parent 1:0 protocol ip prio 4 handle 30 fw flowid 1:30

$TC filter add dev $3 parent 1:0 protocol ip prio 6 handle 20 fw flowid 1:20

$TC filter add dev $3 parent 1:0 protocol ip prio 8 handle 10 fw flowid 1:10

$TC filter add dev $3 parent 1:0 protocol ip prio 10 handle 100 fw flowid 1:100

# ICMP (ip protocol 1) in the interactive class 1:10

# This goes in tc filter priority 3 -- after prioritizing bulk traffic

$TC filter add dev $3 parent 1:0 protocol ip prio 3 u32 match ip protocol 1 0xff flowid 1:10

# To speed up downloads while an upload is going on, put ACK packets in the interactive queue

# This goes in tc filter priority 3 to prevent bulk traffic ACKs from being

# put into the Premium queue priority.  I feel it's okay for Standard and up.

$TC filter add dev $3 parent 1:0 protocol ip prio 3 u32 \

match ip protocol 6 0xff \

match u8 0x05 0x0f at 0 \

match u16 0x0000 0xffc0 at 2 \

match u8 0x10 0xff at 33 \

flowid 1:10

# rest ends up in Standard 1:30

# I'm not sure why we have this rule -- we already set default 30, let's leave it commented out in case we do need it

#$TC filter add dev $3 parent 1: protocol ip prio 10 u32 match ip dst 0.0.0.0/0 flowid 1:30

########## downlink #############

# downlink uses IMQ device and RED qdisc to drop packets coming in too fast

if [ "$1" != "0" ] ; then

# bring up IMQ device

$IP link set imq0 up

$TC qdisc add dev imq0 root handle 1: htb default 30

# main class - max device bandwidth

$TC class add dev imq0 parent 1: classid 1:1 htb rate 100mbit burst 1000k quantum $((${4}+18))

# Exempt class 1:100

$TC class add dev imq0 parent 1:1 classid 1:100 htb rate 100mbit ceil 100mbit burst 1000k prio 1 quantum $((${4}+18))

# root class - max to pipe's max rate

$TC class add dev imq0 parent 1:1 classid 1:2 htb rate $1kbit burst 6k prio 2 quantum $((${4}+18))

# Premium class 1:10

$TC class add dev imq0 parent 1:2 classid 1:10 htb rate $((75*${1}/100))kbit ceil $1kbit burst 6k prio 3 quantum $((${4}+18))

# Express class 1:20

$TC class add dev imq0 parent 1:2 classid 1:20 htb rate $((15*${1}/100))kbit ceil $1kbit burst 6k prio 4 quantum $((${4}+18))

# Standard class 1:30

$TC class add dev imq0 parent 1:2 classid 1:30 htb rate $((10*${1}/100))kbit ceil $1kbit burst 6k prio 5 quantum $((${4}+18))

# Bulk class 1:40

$TC class add dev imq0 parent 1:2 classid 1:40 htb rate 1kbit ceil $1kbit burst 6k prio 6 quantum $((${4}+18))

# add SFQ & RED

$TC qdisc add dev imq0 parent 1:100 handle 100: sfq perturb 10

$TC qdisc add dev imq0 parent 1:10 handle 10: sfq perturb 10

$TC qdisc add dev imq0 parent 1:20 handle 20: red bandwidth $((8*${1}/10))kbit limit 1000000 min 5000 max 100000 avpkt 1000 burst 50 ecn

$TC qdisc add dev imq0 parent 1:30 handle 30: red bandwidth $((7*${2}/10))kbit limit  500000 min 2500 max  50000 avpkt  500 burst 25 ecn

$TC qdisc add dev imq0 parent 1:40 handle 40: red limit 250000 min 1250 max 25000 avpkt 250 burst 10 ecn

# To speed up downloads while an upload is going on, put ACK packets in the inte

$TC filter add dev imq0 parent 1:0 protocol ip prio 4 u32 \

match ip protocol 6 0xff \

match u8 0x05 0x0f at 0 \

match u16 0x0000 0xffc0 at 2 \

match u8 0x10 0xff at 33 \

flowid 1:10

## separate iptables mark into proper classes

## tc filter stops when it finds a match, so rules are reversed here

## in order to make sure we put bulk traffic in its place

$TC filter add dev imq0 parent 1:0 protocol ip prio 9 handle 40 fw flowid 1:40

$TC filter add dev imq0 parent 1:0 protocol ip prio 8 handle 30 fw flowid 1:30

$TC filter add dev imq0 parent 1:0 protocol ip prio 7 handle 20 fw flowid 1:20

$TC filter add dev imq0 parent 1:0 protocol ip prio 6 handle 10 fw flowid 1:10

$TC filter add dev imq0 parent 1:0 protocol ip prio 5 handle 100 fw flowid 1:100

fi