6.3.36.  route [add|drop]  (see below)

There are two major route commands, route add and route drop.

6.3.36.1.  route add <dest addr>[/bits]|default <interface>

[<gateway hostid> [<metric>]]

With no arguments, "route" displays the IP routing table.  "route add"

adds an entry to the routing table, while "route drop" deletes an existing

entry.  "route add" requires at least two more arguments, the host id of

the target destination and the local name of the interface to which its

packets should be sent.  If the destination is not local, the gateway's

host id should also be specified.  (If the interface is a point-to-point

link, then <gateway hostid> may be omitted even if the target is non-local

because this field is only used to determine the gateway's link level

address, if any.  If the destination is directly reachable, <gateway

hostid> is also unnecessary since the destination address is used to

determine the interface link address).  The host names contained in

hosts.net can be used as well as the numeric host ids in these examples.

The optional "/bits" suffix to the destination host id specifies how many

leading bits in the host id are to be considered significant in the

routing comparisons.  If not specified, 32 bits (i.e., full significance)

is assumed.  With this option, a single routing table entry may refer to

many hosts all sharing a common bit string prefix in their IP addresses.

For example, ARPA Class A, B and C networks would use suffixes of /8, /16

and /24 respectively; the command

route add [44]/8 sl0 [44.64.0.2]

causes any IP addresses beginning with "44" in the first 8 bits to be

routed to [44.64.0.2]; the remaining 24 bits are "don't-cares".

When an IP address to be routed matches more than one entry in the routing

table, the entry with largest "bits" parameter (i.e., the "best" match) is

used.  This allows individual hosts or blocks of hosts to be exceptions to

a more general rule for a larger block of hosts.

The special destination "default" is used to route datagrams to addresses

not in the routing table; it is equivalent to specifying a /bits suffix of

/0 to any destination hostid.  Care must be taken with default entries

since two nodes with default entries pointing at each other will route

packets to unknown addresses back and forth in a loop until their time-

to-live (TTL) fields expire.  (Routing loops for specific addresses can

also be created, but this is less likely to occur accidentally).

Here are some more examples of using the route command:

# Most typical, using ax0 and a KISS TNC

route add default ax0

# Station with IP address [44.0.0.10] on local AX.25 channel

# needing an explicit route for some reason

route add [44.0.0.10] ax0

# Route datagrams to IP address 44.0.0.3 to SLIP line #0.

# No gateway is needed because SLIP is point-to point.

route add [44.0.0.3] sl0

# Route packets to IP station [44.0.0.1] using the pseudo vcircuit

# interface, called "vc0"

route add [44.0.0.1] vc0

# Route all default traffic to the gateway on the local Ethernet

# with IP address [44.0.0.1]

route add default ec0 [44.0.0.1]

# The local Ethernet has an ARPA Class-C address assignment;

# route all IP addresses beginning with 192.4.8 to it

route add [192.4.8]/24 ec0

Note:  The metric value is not used by NET.

6.3.36.2.  route drop <dest hostid>

"route drop" deletes an entry from the table.  If a packet arrives for the

deleted address and a default route is in effect, the default will be

used.

Internet Protocol addressing and routing:

Routing is one of the key abilites of tcp/ip. It is important

to understand the basic principles of routing if you are

responsible for a network.

2 types of network computers - Hosts and Gateways.

A "host" is a computer that normally serves as the endpoint

point of a tcp/ip session. It will have only a single

IP address and it will not forward packets to other

machines.

A "gateway" is a machine that normally routes packets, but does

not serve as the users access point to the network. The term

"router" usually refers to a machine designed specifically to

serve as a gateway. Gateways can be built that do not the

software normally associated with end-user usage. For

example, it is possible to build a gateway that does not

have tcp and therefore cannot conduct telnet or ftp

sessions. However, gateways usually run special routing

and network managemnet protocols that may not exist on

hosts.

Of course, there are exceptions to the distinctions

between gateways and hosts. One exception is known

as a "multi-homed host". This is machine running the

software to support end-user usage, but which is attached

to more than one network at the same time. There are many

instances where multi-homed hosts are made to act as

gateways. In addition, routers can have the upper layers

of protocols to support user applications. These mixed

use machines can be problematic.

Cohulip is designed to run as a host. It does not

offer the gateway protocols such as RIP or SNMP. However,

small scale routing can be performed by Cohulip by

putting in a multihomed situation and properly entering

the routing information with the 'ip route' commands.

The Address Scheme:

An internet protocol (IP) address is actually a 32 bit binary

number, though it is usually written as 4 decimal numbers

separated by dots (eg: 192.147.249.146 ). Each of the 4

decimal numbers represents 8 bit (1 byte) of the 32 bit

address.

2 kinds of Addresses, host addresses and network addresses.

Host addresses are generally assigned one to each computer

on a network so that a host's address is significant to

all 32 bits. Network addresses, however, have fewer

significant bits, and this portion of the ip address is

known as the network portion.

Routing setup example 1. A small ethernet with a slip gateway:

Suppose we have an ethernet network that

has 3 hosts with the addresses and names:

192.147.249.158  rwsyss

192.147.249.147  stalo

192.147.249.146  eastern

The network address might be 192.147.249.0

if the network address is significat to 24

bits. If the network address is deemed to be

significant to 16 bits, the network portion of

the address is 192.147. And an 8 bit significant

netwrok address would be 192.

You may hear of the terms class A, class B and class C addresses.

These terms refers to the number of significant bits in the network

address and have implication for routing and network size. A class

A network can have over 16 million computers attached to it, a

class B network can have about 65 thousand hosts and a class C

network has 256 addresses. There can be other sizes of networks

as well, known as 'subnets'.

The above three hosts are actually all addresses on a 16 address

subnetwork. The subnetwork address is 192.147.249.144 and is

significant to 28 bits. This small network occupies a single

segment of ethernet cable, with 192.147.249.158 acting as

a gateway to other portions of the class C network: 192.197.249.000.

-----net192.197.249.000-+---------->The Internet

|

|

| SLIP

|

|

------+---------|192.197.249.158|---net192.197.249.144-----+----ethernet

|          multi-homed-host                          |

|             rwsyss                                 |

|                                                    |

|                                                    |

|                                                    |

|192.197.249.146|                                   |192.197.249.147|

stalo                                                  eastern

The Network bits in the 32 bit address, in this case, are

the first 28 bits and the host bits are the last 4 bits.

The tcp/ip standards dictate that the 'broadcast' address

is reserved. It is defined as the address in which all host

bits are set 1's. In our little subnet, this is 192.197.249.159.

Another concept used in routing is the 'subnet mask'. This is

a number used to extract the network portion of an ip address.

It is defined as having all 1's in the network portion of

the binary address and all 0's in the host portion. For our

example subnet, this would be 255.255.255.240. The cohulip

package doesn't require that the subnet mask be entered. It

is determined from the 'bits' setting in the route commands.

The number of computers on the 144 subnet can be expanded from 3 by

adding 148, 149, 150, 151, 152, 153, 154, 155, 156, 157, 159. However,

there are two addresses that cannot be used for expansion if the

subnet is to remain integral. These are the 'network' address

(144) and the subnet broadcast address (159).

The routing is different for each computer. The idea is simply

to route the packet to the correct hardware facility. stalo and

eastern each have only one ethernet interface, so

on stalo we use the commands:

route add default ne0 [192.149.247.158]

route add [192.149.247.147] ne0

on eastern we use:

route add default ne0 [192.149.247.158]

route add [192.149.247.146] ne0

This will deliver traffic between stalo and eastern directly,

and all other traffic will go through rwsyss.

rwsyss has to be a bit smarter.

route add default sl0

route add [192.149.247.147] ne0

route add [192.149.247.146] ne0

--------------------

Routing Setup 2, an ethenet with 2 gateways,

1 to another ethernet and one to an internet site.

This network is an extended version of the network above,

but altered by adding another ethernet segment. We now have

two acting gateways and a full slate of computers on the

16 address subnet. However, we now have deviced the 16

addresses into two subnets. Stalo is acting as a gateway

between the two ethernets and rwsyss is acting as a

gatway between the local site and the internet. There

are 12 machines on this imaginary network.

-----net192.197.249.000-+---------->The Internet

|

|

| SLIP

|

------------------

|       sl0      |

|                |

|   rwsyss       |

|192.197.249.158 |

|                |

|       ne0      |

------------------

|

|

------+------ net192.197.249.152----------------------------+- << ethernet1

|                                                     |

|                                                     |

|                                                     | support

|                                                     |   and

|                                                     |  sales

-----------------                         -----------------  |    |

|     ne0       |                         |           ne0 |--+    V

|               |                         |               |  |

|192.197.249.153|                         |192.197.249.154|  |

|               |                         |               |  |

|   stalo       |                         |    eastern    |  |

|               |                         -----------------  |

|     ne1       |                                            |

-----------------                                            |

|                                                     |

R&D    | << ethernet2 192.197.249.144                        |

|     |                                                     |

V     |  -----------------                                  |

+--| ne0           |                                  |

|  |               |                                  |

|  |192.197.249.145|                                  |

|  |               |                                  |

|  |     jobo      |                                  |

|  -----------------                                  |

|                                  -----------------  |

|                                  |           ne0 |--+

|                                  |               |  |

|                                  |192.197.249.155|  |

|                                  |               |  |

|                                  |    western    |  |

|                                  -----------------  |

|  -----------------                                  |

+--| ne0           |                                  |

|  |               |                                  |

|  |192.197.249.146|                                  |

|  |               |                                  |

|  |     jackal    |                                  |

|  -----------------                                  |

|                                  -----------------  |

|                                  |           ne0 |--+

|                                  |               |  |

|                                  |192.197.249.156|  |

|                                  |               |  |

|                                  |    northern   |  |

|                                  -----------------  |

|  -----------------                                  |

+--| ne0           |                                  |

|  |               |                                  |

|  |192.197.249.147|                                  |

|  |               |                                  |

|  |     jill      |                                  |

|  -----------------                                  |

|                                  -----------------  |

|                                  |           ne0 |--+

|                                  |               |  |

|                                  |192.197.249.157|

|                                  |               |

|                                  |    southern   |

|                                  -----------------

|  -----------------

+--| ne0           |

|  |               |

|  |192.197.249.148|

|  |               |

|  |     jimmy     |

|  -----------------

|

|

|

|

|

|

|

|  -----------------

+--| ne0           |

|  |               |

|  |192.197.249.149|

|  |               |

|  |     jelly     |

|  -----------------

|

|

|

|

|

|

|

|  -----------------

+--| ne0           |

|  |               |

|192.197.249.150|

|               |

|     jyro      |

-----------------

This type of subnetting the ethernets allows each ethernet

to run at a greater speed for local traffic. We have the

support and sales people on one ethernet, directly connected

to rwsyss in order to handle telnet sessions with users.

On the other ethernet, we have the 'j' machines in the

R&D department with 'jyro' acting as a file server, and

stalo acting as a gateway.

Suppose the file server 'jyro' needs to be accessible by

everyone in the 'j' group, but contains proprietary secrets.

It needs to be shielded from the internet. Stalo can be

set up as a 'firewall'. In this case, no packets would

be transfered from the 'j' net through stalo. Instead,

stalo could run 2 simultaineous copies of cohulip. If

someone in the 'j' group needs to ftp in a file or take

a telnet session over the internet, she would use her local

telnet to log onto stalo's 'j' copy and from there using

a stalo login shell, do the telnet or ftp etc. This completely

isolates the 'j' net so that jyro may freely supply files to

the 'j' group, without there being a concern that anyone on

the internet could read or tamper with jyro.

It also leaves the sales and support staff as accessible by

telnet as they would be by voice telephone. If the address

space were expanded further, there could be similar secure

and/or open subnets for the likes of payroll dept., planning,

purchases etc.

Routing commands:

On the 'j' network, each machine except stalo will

have a default route specified as:

route add ne0 default

The result is that all packets will go out over the

ethernet interface on these machines. There are two copies

of net on stalo. On the 'j' subnet copy, the only routing

command is:

route add ne1 default.

On the 152 network, the routing tables resemble

those in our previous example. Each host will have a direct

route entry for the other hosts and they will also have a

default route to the slip gateway. The copy of cohulip

connecting to the 152 network will have the same style

of routing tables.

The Gateway will have a simple, but more extensive

routing setup:

route add default sl0

route add [192.149.247.152]\28 ne0

As you can see, rwsyss does not even know of the existence

of the 'j' net behind the firewall on stalo. The use of stalo

as a firewall requires that external sockets be enabled on the

152 copy of cohulip, other wise, only one person on 'j' net

can use external services at a time.

----------